- First-Time Setup

- Initially, no option is selected; one of the modes must be configured.

- Exit: Clicking to closes the program without making a selection.

- Battery Status Checking: Suitable for PCs not powered by EPB-1001, preventing unwanted shutdown actions and avoiding the creation of unnecessary startup items.

- Smart Battery Management: Suitable for PCs powered by EPB-1001; by default, it creates a startup item, minimizes to the notification area, and may trigger shutdown behaviors based on the System Shutdown Warning settings.

- Settings can be modified later on the Information page.

- CLI Silent Setup commands:

- Battery Status Checking: BatteryManagement.exe -S BSC

- Smart Battery Management: BatteryManagement.exe -S SBM

- Related files, the configuration file (.cfg) is automatically generated after the program is launched.

- Notification Area (System Tray) Operations & Status Icons

- When minimized to the notification area, you can double-left-click the icon or right-click and select Show to open the main program window.

- Right-clicking the icon and selecting Exit, or clicking the Exit button on the Information page, will close the program.

- Detecting icon: Displayed during detection as shown below.

- On Battery icon: Displayed when running on battery; the battery icon reflects the higher remaining capacity among the detected batteries.

- Charging icon: Displayed during battery charging as shown below.

- Plugged In icon: Displayed when no battery is inserted and only the EPB-1001 external power supply is used.

- Sleep icon: Displayed when Sleep-related settings are enabled and triggered.

- Battery Level Reminder

- Default: Enabled / 20%

- Range: 10% to 100%

- Low Battery Shutdown

- Default: Enabled / 10% / 1 min

- Range: 6% to 10% / 1 min to 5 min

- Battery Charging Reminder Window: Battery 1 and Battery 2 will trigger individual reminders respectively.

- Low Battery Shutdown Window: The prompt messages vary slightly depending on whether External Monitoring or Built-in Monitoring is used.

- Windows Shutdown Prompt: The default Windows warning window displayed when a shutdown action is triggered.

- Windows Cancellation Prompt: The default Windows notification window displayed when a scheduled shutdown action is canceled